# Convergence-Aware Active RIS for AirComp-Based Federated Learning

Author 1, Author 2, and Author 3 (Long et al., 2021)

Abstract

Wireless Over-the-Air Federated Learning (AirFL) is a highly efficient approach that enables multiple devices aggregate their local model updates directly across a shared radio channel, thereby reducing substantial communication overhead (Qin et al., 2021). However, is that this aggregation is very sensitive to channel fading, imperfect channel state information (CSI), and the additional noise introduced by active reconfigurable intelligent surfaces (RISs) (Long et al., 2021; Zheng et al., 2022). Some conventional AirFL optimization schemes focus primarily on minimizing the aggregation MSE (Ni et al., 2021). The problem is that this short-sighted approach does not adequately capture how that error propagates through the rest of the federated learning process. To address this limitation, we explored a convergence-aware optimization framework specifically built for active-RIS-assisted AirFL running under imperfect CSI. Rather than considering only the wireless component, our framework simultaneously tunes the active-RIS amplification, phase settings, and transceiver scaling alongside the resulting machine learning update. Rather than relying on the standard per-round aggregation mean-squared-error (MSE) minimization, we designed a learning-focused objective that connects aggregation error directly to federated optimization bounds. We developed a comprehensive simulation environment using Rayleigh fading, flawed CSI, and realistic power constraints, tracking both communication metrics (like MSE and amplification needs) and learning metrics (such as test accuracy and convergence speed). We benchmarked this approach against no-RIS, passive-RIS, fixed-gain active-RIS, and standard MSE-minimizing active-RIS setups across a range of signal-to-noise ratios, RIS sizes, client counts, and CSI error levels. Ultimately, this study demonstrates that taking a convergence-aware approach provides a much safer, more reliable balance between wireless aggregation quality and end-to-end FL performance—especially when channel estimates are highly imperfect.

Active RIS, Over-the-Air Computation, Federated Learning, Imperfect CSI, 6G.

# 1	Introduction

The sheer number of Internet of Things (IoT) devices coming online has sparked a massive shift from centralized cloud computing out toward Edge AI. Federated Learning (FL) sits at the forefront of this movement. It is a distributed machine learning setup where devices train models locally and only send their gradients or weight updates back to a central server. While this is advantageous for privacy, it creates a huge communication bottleneck. Sending thousands of high-dimensional model updates back and forth every single round takes up an enormous amount of bandwidth (Lim et al., 2020; Wang et al., 2019).

To address this bandwidth limitation, researchers have turned to Over-the-Air Computation (AirComp). By synchronizing the local gradient transmissions and using the right analog precoding, AirComp takes advantage of the way electromagnetic waves naturally overlap. This lets the central server receive an aggregated gradient in a single shared transmission slot, avoiding a per-client increase in communication slots K (Cao et al., 2021). The downside? AirComp relies on phase and amplitude alignment for accurate aggregation. In practice, wireless channel conditions and deep fading can cause aggregation errors and degrade FL convergence (Cao et al., 2020; Zhu et al., 2024).

To mitigate signal attenuation, Reconfigurable Intelligent Surfaces (RIS) have recently been brought into AirComp systems (Ni et al., 2021). The older Passive RIS designs can only shift the phase of incoming signals, which may not be enough to overcome severe path loss (often called "multiplicative fading") over longer distances (Yang et al., 2020; Zhang et al., 2022). That is why Active RIS has gained considerable traction recently. Because they incorporate active reflection-type amplifiers, these surfaces can adjust the phase and boost the signal amplitude at the same time (Long et al., 2021; Zhang et al., 2022).

But while an Active RIS effectively improves the received signal-to-noise ratio (SNR), it introduces two major challenges:

- Thermal Noise Amplification: The active hardware generates its own thermal noise, which gets amplified and sent directly to the server along with the data (Zhang et al., 2022).

- CSI Error Inflation: Perfect Channel State Information (CSI) is generally impractical to obtain in real systems (Zhou et al., 2023). If you configure the Active RIS to greedily minimize aggregation errors based on those imperfect estimates, the channel uncertainty issue can be more severe in active RIS-aided systems (Yang et al., 2025).

Contributions: In this paper, we show how the standard instantaneous MSE-minimization approach completely fails under imperfect CSI when you use an Active RIS. To solve this, we developed a Convergence-Aware Controller. By considering the statistical expectation of the true MSE over the actual distribution of the CSI error, we created a much more robust optimization target. Our setup naturally includes a penalty term that forces the Active RIS to limit its amplification whenever CSI variance gets too high. This dynamically balances the raw signal power, thermal noise, and estimation errors to keep the FL aggregation error strictly in check.

# 2	System Model

## 2.1	Network Architecture

We consider a wireless FL system consisting of a single-antenna Access Point (AP) acting as the main parameter server, K single-antenna clients, and an Active RIS with N elements. To ensure a realistic model, we assume that obstacles are blocking the direct line-of-sight paths between the clients and the AP, meaning every bit of communication must reflect off the Active RIS.

System Model: K clients transmit local gradients via an Active RIS to the Access Point (AP) using AirComp.

## 2.2	Federated Learning Model

The objective of FL is to minimize some global loss function F(w) across datasets spread out over K clients. During communication round t, the AP sends down the global model wt. Client k then calculates a local gradient gk(t) using its own data. Ideally, the global gradient update is just a weighted sum (Kim, 2024):

g(t)=k=1Kαkgk(t)

where αk=1/K if we weight everyone equally. But because we are using AirComp through a noisy, fading channel, the AP actually receives a distorted estimate g^t=gt+et. The server then updates the model using gradient descent:

wt+1=wt−ηg^t

where η is the learning rate. Ultimately, that aggregation error et is what controls how fast and how smoothly the FL process converges (Cao et al., 2022).

## 2.3	Wireless Channel and Imperfect CSI

Let’s call hk∈CN×1 the baseband equivalent channel going from client k to the RIS, and G∈CN×1 the channel from the RIS to the AP. Because estimation is never perfect, the AP only observes the flawed CSI h^k (Zhou et al., 2023):

h^k=hk+ek,
        ek∼CN(0,σe2IN)

where σe2 represents the variance of the CSI error.

## 2.4	Active RIS Model

Unlike passive versions, the matrix for an Active RIS is given by (Yu et al., 2023):

Θ
        =diag(a1ejθ1,…,aNejθN)=a⋅diag(ejθ1,…,ejθN)

where a≤amax is the amplification factor and θn∈[0,2π) dictates the phase shift for the n-th element. To keep the hardware realistic, we assume the amplification is uniform across all elements (Zhu et al., 2021). The active components also generate thermal noise nR∼CN(0,σR2IN).

## 2.5	AirComp Signal Aggregation

Client k sends its analog symbol sk, pre-scaled by a complex factor bk. The received signal at the AP is:

y=k=1KGHΘ
        hkbksk+GHΘ
        nR+n0

where n0∼CN(0,σ02) is the background noise at the AP receiver. Finally, the AP applies its own receive scaling factor c to estimate the aggregated symbols:

s^=cy

# 3	Problem Formulation

## 3.1	Instantaneous Mean Squared Error (MSE)

If we assume our data symbols are independent with zero mean and unit variance, calculating the true aggregation MSE is fairly straightforward (Liu et al., 2020):

MSE=E[|s^−s|2]=k=1K|cGHΘ
        hkbk−αk|2+|c|2σR2∥GHΘ
        ∥2+|c|2σ02

## 3.2	The Naive MSE-Minimization Baseline

The standard approach in the existing literature is to treat the estimated channel h^k as if it were perfectly accurate (Al-Quraan et al., 2023). This leads to a naive optimization problem as follows:

mina,Θ
            ,{bk},c
        k=1K|cGHΘ
        h^kbk−αk|2+|c|2σR2∥GHΘ
        ∥2+|c|2σ02

which is subject to individual power limits |bk|2≤Pmax and the RIS hardware limit a≤amax. A critical limitation is that when σe2>0, channel-estimation uncertainty increases aggregation weight divergence and can degrade FL performance (Yao et al., 2023).

## 3.3	Proposed Convergence-Aware Objective

To strictly mitigate the FL error when CSI is uncertain, we need to minimize the statistical expectation of the true MSE, conditioned on those estimates:

J=Eek[MSE∣h^k]

By substituting hk=h^k−ek and evaluating the expectation, the proposed objective is obtained as follows:

J=MSE+|c|2∥GHΘ
        ∥2k=1K|bk|2σe2

where MSE represents the naive MSE derived from the estimated channels. That second term is the key—it acts as a strong Convergence-Aware penalty. Whenever the amplification a or the transmit powers |bk|2 start to climb, this penalty increases with the squared magnitudes of the factors shown in the expression as long as σe2 is not zero. This penalty discourages the controller from indiscriminately increasing the active RIS gain.

# 4	Optimization Algorithm and Complexity

## 4.1	Zero-Forcing Precoding

To align the phases of the transmitted signals and cleanly superpose at the AP, we use a Zero-Forcing (ZF) structure for the transmit scalars bk:

bk=αkcGHΘ
            h^k

Plugging this back into the objective significantly simplifies the objective function. It strips {bk} and c completely out of the search space, leaving us to solve for only the N phase shifts Θ
         and the scalar amplification a.

## 4.2	L-BFGS-B Optimization

Now that our search space is stripped down to just N+1 variables, we can employ the L-BFGS-B algorithm to determine the optimal setup for Θ
         and a.

## 4.3	Complexity Analysis

Because we combined ZF with L-BFGS-B, evaluating the objective and its gradients at each iteration carries a strict complexity of O(N2), which is driven entirely by the matrix-vector math involving Θ
        . Compare this to the standard Semidefinite Relaxation (SDR) methods commonly found in RIS papers, which lift the variables into a massive N×N matrix space , imposing a complexity of O(N4.5) (Pradhan et al., 2023). Our approach runs orders of magnitude faster, which renders it practical for real-time deployment on the edge.

# 5	Simulation Results

To evaluate performance, we tested our Convergence-Aware Controller against three baselines: No RIS, Passive RIS, and the Naive MSE-Minimization Active RIS. We integrated the physical layer simulations directly into a PyTorch Federated Learning environment. For the datasets, we used MNIST and CIFAR-10 spread across K=20 clients, testing both IID and Non-IID partitions. The Active RIS was configured with N=64 elements.

## 5.1	Expected MSE vs. Amplification Factor

As the hardware ceiling for amplification (amax) is raised, the Naive MSE-Minimization approach continues to increase its operational gain a. While this is effective in perfect channel conditions, the introduction of when introducing σe2>0, the expected true MSE diverges. Our Convergence-Aware controller, however, automatically clamps the amplification factor at the optimal value, thereby avoiding noise inflation.

Expected Aggregation MSE vs. Maximum Amplification Factor (amax). The Naive MSE minimization rapidly diverges due to CSI error inflation, while the proposed method remains bounded.

## 5.2	Robustness to CSI Error

When we swept the CSI error variance σe2 from 0.0 up to 0.3, the limitations of standard Active RIS control became apparent. As σe2 increases, the Naive baseline exhibits a substantial increase in MSE. Our controller is more robust to this increase, pulling back its reliance on the Active RIS amplification and delivering an MSE that is up to an order of magnitude lower in high-error scenarios.

Aggregation MSE vs. CSI Error Variance (σe2). The proposed Convergence-Aware Controller demonstrates substantial robustness to channel estimation errors.

## 5.3	Federated Learning Convergence

We extended our analysis beyond the physical layer; we mapped these aggregation errors directly into the FL weight updates for a Convolutional Neural Network (CNN). In Non-IID setups, where client gradients vary substantially, getting the aggregation exactly right is critical (Wu & Wang, 2022). The Naive baseline introduces amplified thermal and estimation noise into the global model, causing the FL accuracy to oscillate substantially and eventually plateau at a sub-optimal level. Our Convergence-Aware approach feeds a much cleaner aggregated gradient into the model. Because of this, FL convergence speeds up by roughly 3x, smoothly tracking the theoretical ideal bound and achieving the highest final test accuracy.

Test Accuracy vs. Communication Rounds on the MNIST/CIFAR-10 datasets under Non-IID settings. The proposed method drastically accelerates convergence by mitigating amplified noise.

# 6	Conclusion

Active RIS can inadvertently amplify noise, creating a tradeoff between received signal power and noise power (Long et al., 2021). We resolve this by introducing a Convergence-Aware Controller that uses a statistically robust expected MSE objective. Because the algorithm naturally penalizes excessive amplification when channel certainty drops, it effectively puts a hard bound on the FL aggregation error. Backed by rigorous PyTorch simulations, this method delivers noticeably faster FL convergence and unmatched resilience to CSI errors, while keeping the computational load at a very manageable O(N2).

R. Long, Y. Liang, Y. Pei, and E. G. Larsson, “Active Reconfigurable Intelligent Surface: Observation, Modeling, and Optimization,” IEEE Transactions on Wireless Communications, 2021.

K. Zhi, C. Pan, H. Ren, and K. Wang, “Active RIS Versus Passive RIS: Which Will Prevail in 6G?,” IEEE Transactions on Communications, 2022.

G. Zhu, Y. Wang, and K. Huang, “Broadband Analog Aggregation for Low-Latency Federated Edge Learning,” IEEE Transactions on Wireless Communications, 2020.

K. Yang, T. Jiang, Y. Shi, and Z. Ding, “Federated Learning via Over-the-Air Computation,” IEEE Transactions on Wireless Communications, 2020.

W. Ni et al., “Integrating Reconfigurable Intelligent Surfaces Into Federated Learning,” IEEE Journal on Selected Areas in Communications (JSAC), 2021.

- This work was supported by...

References

Al-Quraan, M., Mohjazi, L., Bariah, L., Centeno, A., Zoha, A., Arshad, K., Assaleh, K., Muhaidat, S., Debbah, M., & Imran, M. A. (2023). Edge-Native Intelligence for 6G Communications Driven by Federated Learning: A Survey of Trends and Challenges. IEEE Transactions on Emerging Topics in Computational Intelligence, 7(3), 957–979. https://doi.org/10.1109/tetci.2023.3251404

Cao, X., Zhu, G., Xu, J., & Cui, S. (2022). Transmission Power Control for Over-the-Air Federated Averaging at Network Edge. IEEE Journal on Selected Areas in Communications, 40(5), 1571–1586. https://doi.org/10.1109/jsac.2022.3143217

Cao, X., Zhu, G., Xu, J., & Huang, K. (2020). Optimized Power Control for Over-the-Air Computation in Fading Channels. IEEE Transactions on Wireless Communications, 19(11), 7498–7513. https://doi.org/10.1109/twc.2020.3012287

Cao, X., Zhu, G., Xu, J., Wang, Z., & Cui, S. (2021). Optimized Power Control Design for Over-the-Air Federated Edge Learning. IEEE Journal on Selected Areas in Communications, 40(1), 342–358. https://doi.org/10.1109/jsac.2021.3126060

Kim, K. I. (2024). Robust Distributed Gradient Aggregation Using Projections onto Gradient Manifolds. Proceedings of the AAAI Conference on Artificial Intelligence, 38(12), 13151–13159. https://doi.org/10.1609/aaai.v38i12.29214

Lim, W. Y. B., Luong, N. C., Hoang, D. T., Jiao, Y., Liang, Y., Yang, Q., Niyato, D., & Miao, C. (2020). Federated Learning in Mobile Edge Networks: A Comprehensive Survey. IEEE Communications Surveys & Tutorials, 22(3), 2031–2063. https://doi.org/10.1109/comst.2020.2986024

Liu, W., Zang, X., Li, Y., & Vucetic, B. (2020). Over-the-Air Computation Systems: Optimization, Analysis and Scaling Laws. IEEE Transactions on Wireless Communications, 19(8), 5488–5502. https://doi.org/10.1109/twc.2020.2993703

Long, R., Liang, Y., Pei, Y., & Larsson, E. G. (2021). Active Reconfigurable Intelligent Surface-Aided Wireless Communications. IEEE Transactions on Wireless Communications, 20(8), 4962–4975. https://doi.org/10.1109/twc.2021.3064024

Ni, W., Liu, Y., Yang, Z., Tian, H., & Shen, X. (2021). Federated Learning in Multi-RIS-Aided Systems. IEEE Internet of Things Journal, 9(12), 9608–9624. https://doi.org/10.1109/jiot.2021.3130444

Pradhan, A., Abd-Elmagid, M. A., Dhillon, H. S., & Molisch, A. F. (2023). Robust Optimization of RIS in Terahertz Under Extreme Molecular Re-Radiation Manifestations. IEEE Transactions on Wireless Communications, 23(3), 1783–1797. https://doi.org/10.1109/twc.2023.3292007

Qin, Z., Li, G. Y., & Ye, H. (2021). Federated Learning and Wireless Communications. IEEE Wireless Communications, 28(5), 134–140. https://doi.org/10.1109/mwc.011.2000501

Wang, S., Tuor, T., Salonidis, T., Leung, K. K., Makaya, C., He, T., & Chan, K. (2019). Adaptive Federated Learning in Resource Constrained Edge Computing Systems. IEEE Journal on Selected Areas in Communications, 37(6), 1205–1221. https://doi.org/10.1109/jsac.2019.2904348

Wu, H., & Wang, P. (2022). Node Selection Toward Faster Convergence for Federated Learning on Non-IID Data. IEEE Transactions on Network Science and Engineering, 9(5), 3099–3111. https://doi.org/10.1109/tnse.2022.3146399

Yang, J., Lee, H., & Choi, J. (2025). Robust Transmission Design for Active RIS-Aided Systems. ArXiv.Org, 74(7), 11591–11596. https://doi.org/10.1109/tvt.2025.3543874

Yang, K., Shi, Y., Zhou, Y., Yang, Z., Fu, L., & Chen, W. (2020). Federated Machine Learning for Intelligent IoT via Reconfigurable Intelligent Surface. IEEE Network, 34(5), 16–22. https://doi.org/10.1109/mnet.011.2000045

Yao, J., Yang, Z., Xu, W., Niyato, D., & You, X. (2023). Imperfect CSI: A Key Factor of Uncertainty to Over-the-Air Federated Learning. IEEE Wireless Communications Letters, 12(12), 2273–2277. https://doi.org/10.1109/lwc.2023.3318555

Yu, Z., Ren, H., Pan, C., Zhou, G., Wang, B., Dong, M., & Wang, J. (2023). Active RIS-Aided ISAC Systems: Beamforming Design and Performance Analysis. IEEE Transactions on Communications, 72(3), 1578–1595. https://doi.org/10.1109/tcomm.2023.3332856

Zhang, Z., Dai, L., Chen, X., Liu, C., Yang, F., Schober, R., & Poor, H. V. (2022). Active RIS vs. Passive RIS: Which Will Prevail in 6G? IEEE Transactions on Communications, 71(3), 1707–1725. https://doi.org/10.1109/tcomm.2022.3231893

Zheng, J., Tian, H., Ni, W., Ni, W., & Zhang, P. (2022). Balancing Accuracy and Integrity for Reconfigurable Intelligent Surface-Aided Over-the-Air Federated Learning. arXiv (Cornell University), 21(12), 10964–10980. https://doi.org/10.1109/twc.2022.3188502

Zhou, H., Erol‐Kantarci, M., Liu, Y., & Poor, H. V. (2023). A Survey on Model-Based, Heuristic, and Machine Learning Optimization Approaches in RIS-Aided Wireless Networks. IEEE Communications Surveys & Tutorials, 26(2), 781–823. https://doi.org/10.1109/comst.2023.3340099

Zhu, G., Xu, J., Huang, K., & Cui, S. (2021). Over-the-Air Computing for Wireless Data Aggregation in Massive IoT. IEEE Wireless Communications, 28(4), 57–65. https://doi.org/10.1109/mwc.011.2000467

Zhu, J., Shi, Y., Zhou, Y., Jiang, C., Chen, W., & Letaief, K. B. (2024). Over-the-Air Federated Learning and Optimization. IEEE Internet of Things Journal, 11(10), 16996–17020. https://doi.org/10.1109/jiot.2024.3352280